Order form of RRP Super ROM ECU

Dear Customer :
Please fill the information we need for each modification of RRP Super ROM ECU re-program to your car as follow.

| Name |  |
| --- | --- |
| Address |  |
| Phone number |  |
| E-mail |  |
| Year of your car |  |
| Body number | ZC31S - |
| ECU number | 33920 - |
| Spec of your car (please inform us the modification of your car) | Muffler : Catalyst : Air cleaner : Throttle : Other : |
| Type of Swift | Type 1 Type 2 |
| Requirement if any |  |

Correspond list of ECU number :

| ECU number |  | Type | Transmission |
| --- | --- | --- | --- |
| 33920 | 57K00 | Type 1 | Manual |
|  | 57K01 | Type 1 | Manual |
|  | 57K02 | Type 1 | Manual |
| 33920 | 57K10 | Type 1 | Automatic |
|  | 57K12 | Type 1 | Automatic |
|  | 57K14 | Type 1 | Automatic |
| 33920 | 57K40 | Type 2 | Manual |
|  | 57K41 | Type 2 | Manual |
|  | 57K42 | Type 2 | Manual |
|  | 57K43 | Type 2 | Manual |
|  | 57K51 | Type 2 | Manual |
| 33920 | 57K53 | Type 2 | Automatic |

Please check your ECU number reference with the picture

[ Method of taking your ECU out from your car and reset ECU to your car ]

- Open the engine bay and take the battery terminal minus (-) from a battery. When you take it out, please check the engine stopped and the ignition key should be OFF position.

- After 1 minute later, please take ECU out from next to the battery.

- Please take care with handling and wrap ECU with special protection materials to send us your ECU.

- After receiving your ECU from us or receiving the new ECU from us, please take battery terminal minus (-) out from the battery then replace ECU to your car.

- After finishing the wiring of ECU, please connect the battery terminal minus (-) to the battery.

- Turn the ignition key to ON position (Note : Don’t start the engine !) and wait for 30 seconds.
Once again turn the ignition key to OFF position and wait for 30 seconds again.
And then, please start the engine.

- If you don’t find any trouble at this moment, the setting ECU is done.

All copyright (C) Reserved by R's inc Japan 2008